NAME:    RESHMA.P

REG NO: 17MIS1009

LAB-3:    FIREWALL

Step-1: Network Topology

Step-2: Configuration  of ip

i)Configuration of asa firewall

a)remove default dchp

Delete previous dhpc address

Previous dhpc successfully deleted

Command Explanation:

Use the machine connected to the ASA, and go to its terminal.

- En

- (no password by default, so just hit [enter])

- show run

  - It might have some preconfigured IPs set up already, so we need to check first.

  - Interface vlan might have shit in it, and the dhcp is enabled on the firewall by default so we need to remove those default settings

- config t

- no dhcpd [whatever dhcp address it had as default]

  - Removes the dhcp config

- ctrl+c

b)configure vlan 1 and vlan 2

Configure vlan 1 and vlan 2. This will override previous vlan1 and vlan2 configuration prefixed  in cisco packet tracer

Command Explanation:

Inside

- config t

- int vlan 1

  - In the int (interface) vlan1 (where the preconfigured IPs were seen when we did show run)

- ip add [our inside ip] [our mask] (here, 192.168.1.1 255.255.255.0 for inside / private network)

- no shut

- nameif inside

- security-level 0

- exit

- int [what physical port is connected for our inside firewall] (here, et0/0)

- switchport access vlan 1

  - So whatever setting we did in vlan 1 (setting IP, security level 100, renaming to inside) is now working on that et0/0 port

- exit

- Ctrl+c

Outside

- Config t

- Int vlan 2

- Ip add [our outside ip] [our mask] (here, 33.2.1.2 255.0.0.0 for outside / public network)

- No shut

- Nameif outside

- Security-level 100

- Exit

- Int [what physical port is connected for our outside firewall] (here, et0/1)

- Switchport access vlan 2

- Exit

- Ctrl + c

ii) router

Set ip and subnest mask for et0/0  and et0/1 for router

Equivalent Commands in cli and it’s explanation:

Go to the CLI

- En

- Int gig0/0

- Ip add [router in ip] (here, 33.2.1.1 255.0.0.0)

- No shut

- Exit

- Int gig0/1

- Ip add [router out ip] (here, 9.9.9.1)

- No shut

- Ctrl+c

Step 3:  Set up DHCP and DNS Server on the ASA

set up dhcp address and dns

now go to pc in network and in desktop choose dhcp

Command Explanation:

Open up Terminal on Server console

- Conf t

- Dhcpd address [inside ip range for network] (here, let's say 192.168.1.10-192.168.1.30) inside

  - The range depends on the number of computers you want in your network

  - 10 - 30 is the range we specified here

- Dhcpd dns 9.9.9.9 interface inside

  - Basically configuring the DNS Ip on the dhcp server

- exit

Set DHCP on for the PCs and see if it works

Step 4:Configure default route on ASA

configure route

Command Explanation:

Just for easy traversal of packets without direction, we create a forwarding route.

Open up terminal again

- Config t

- Route outside 0.0.0.0 0.0.0.0 33.2.1.1

  - Any IP

  - Any subnet

  - 33.2.1.1 is the IP of the router

Step 5: Configure Dynamic Route using OSPF Routing protocol

Configure dynamic route

Command Explanation:

Go to router's CLI

- Config t

- Ospf 1

  - 1 - Process ID

- Net 33.0.0.0 0.255.255.255 area 0

  - Networks connected? 33 and 9

  - 0.255.255.255 means anything with 33.x.x.x IP is included

    - If it was 33.1.0.0 0.0.255.255, anything with 33.1.x.x IP is included

- Net 9.0.0.0 0.255.255.255 area 0

Step 6:  Configuring NAT

Configure NAT

Command Explanation:

To make things easier to manage, we throw all our objects together into an object network and configure a way for the inside network to communicate with the outside using NAT

Terminal of the ASA:

- Config t

- Object network [name of object network]

- Subnet [subnet IP] [subnet mask]

  - Here, our private network runs on IP 192.168.1.0

  - So, our subnet can be 192.168.1.0 and the mask can be 255.255.255.0 or even 192.168.0.0 255.255.0.0 or even 192.0.0.0 255.0.0.0

- Nat (inside, outside) dynamic interface

  - NAT - Network Address Translation. Network address translation is a method of remapping an IP address space into another by modifying network address information in the IP header of packets while they are in transit across a traffic routing device. So, we basically connect the private and public ips.

- Exit

Step 7: ACL on the firewall

ACL on firewall

Command Explanation:

Terminal of ASA:

- Config t

- Access-list [nameoflist] extended permit tcp any any

  - Any source

  - Any destination

- Access-list [nameoflist] extended permit icmp any any

  - Allows icmp packets too

- Access-group [nameoflist] in interface outside

  - In for input traffic

  - Out for output traffic

  - Activates ACL and gives input traffic to the outside network

Step 7: sending packets to check final result

Send packet from pc0 to router, pc1 to router, pc2 to router ; if its successful then we got the desired result